Total gas treatment volume      1200 SLM

(H2 treatment volume: up to 120 SLM)

Heating chamber temperature   800~850℃

Inlet port                     ISO 63 flange, 1 port

Outlet port                    ISO 200 flange, 1 port, -25~-75 mmH2O

Cabinet extraction port         Diameter 100 mm flange, -25~-125 mmmH2O

Waste water drain port         1/2 inch lok-fitting

Electrical power               20 KW

Voltage/Frequency             208 VAC / 50/60Hz

Phase                        3 phase

Rating current                75 A

Full load current              55 A

Running current              30~40 A

PCW IN, OUT               2~3 kgf㎠, 8~12 SLM, 3/8 inch lock-fitting

CDA(Air)                2~3 kgf㎠, Max. 750 SLM, 1 inch lock-fitting

Nitrogen                    4~6 kgf㎠, Max. 100 SLM, 1/4 inch lock-fitting

Outlet gas sampling port       1/4 inch lock-fitting

Dimension                   1800(W) ×900(D) ×2000(H)

Weight                      1200 Kg